Lab Report – 8

Name: Aryan Gupta

Roll No.:2021113012

Group Number: 8

Part-A

Aim: To verify the working of a tri-state buffer.

Components Used: Arduino Uno, wires, LED, resistor, given implementation of a tri-state buffer.

Procedure:

- Give enable and input to the buffer from Arduino.

- Tabulate the output for various values of enable and input.

Theory :

The tri-state buffer (Fig 1) functions just as a regular digital buffer where the value at its input is propagated to its output. But it has an additional capability that allows us to configure its output to a Hi-Z (high impedance) state.

When the output of the buffer is in Hi-Z state it is basically disconnected (isolated) from the rest of the electric circuit. This makes it very useful when connecting multiple devices on a single bus, as its isolation prevents the occurrence of a short circuit event.

Reference Circuit:

Conclusion: The tri-state buffer works as expected.

Tinkercad Link:

https://www.tinkercad.com/things/dZOb0D53WFm-l8-p1/editel?sharecode=liCNLA0O8pX9peZUhvOPEH1rUzz-uchrvkjSI8RkoAU

Part-B

Aim: To manage the flow of data between 2 shift registers using tri-state buffers.

Components Used: Arduino Uno, wires, LEDs, resistors, given implementation of a tri-state buffer.

Reference Circuit:

Procedure:

- Give input, OR clock and SR clock to the first register using Arduino.

- Connect each output of the first register to a buffer.

- Connect those buffers to the input of the second register.

- Give OR clock and SR clock to the second register using Arduino.

- Connect the enables of the buffers to Arduino.

- Enable each buffer one by one in sync with the SR clock of the second

register to convert the parallel output of the first register into a serial input for the second register.

Conclusion: The circuit works as expected. The output of second register is the same as expected output from the first register with the given input.

Conclusion:

From this experiment, we achieved the passage of data(output) of one register as a input to the another register via bus and understood the data flow control using tristate buffers.

(O1-O3 are leds of shift register 1 & 2)

Tinkercad Link:

https://www.tinkercad.com/things/7bTd0cKvnNo-copy-of-data-transfer-using-tri-state-buffer/editel?sharecode=qsa0bSjJyxvu9DZh2CP27eo8UqE0DntZMSpeRH3CDY8